🦾  COMPLETE BEGINNER'S GUIDE

Design & Development of a

Dual-Mode Vision-Guided Robotic Arm

Every concept explained from scratch — 2025-2026

What this guide covers:

- What a Robotic Arm actually is and how it moves

- ROS2 — the operating system for robots

- Gazebo — the physics simulation world

- URDF / Xacro — how robots are described in code

- ros2_control — making joints move

- Computer Vision with OpenCV

- Camera Calibration & real-world coordinate math

- YOLOv8 — AI-powered object detection

- MediaPipe — hand gesture recognition

- Python OOP — the coding style this project uses

- MQTT — the internet messaging system for remote control

- Arduino — the hardware brain

- Week-by-week learning roadmap

# SECTION 0 — The Big Picture (Read This First!)

Before diving into any technical concept, it's essential to understand what you are actually building and why each piece exists. Think of this section as the 'map' you'll keep referring back to.

## 0.1  What Are You Building?

You are building a robotic arm that can do three things entirely on its own:

- AUTONOMOUS MODE: The robot watches a conveyor belt through a camera. It sees objects, figures out what colour or type they are using AI, and automatically picks them up and drops them in the right bin — no human needed.

- LOCAL GESTURE MODE: You wave your hand in front of your webcam. The robot mirrors your hand's movement in real time. Raise your hand — the arm goes up. Twist your wrist — the arm rotates.

- REMOTE GESTURE MODE: Someone anywhere in the world opens a web page on their phone. The phone camera tracks their hand. Those hand movements are sent over the internet and the robot arm moves accordingly.

The system automatically detects which mode to be in. If it sees a hand in front of the camera, it hands control to the human. If not, it keeps sorting autonomously. No buttons needed.

## 0.2  The Technology Stack (Simplified)

Here is every technology this project uses, grouped by what it does:

| ROS2 Jazzy | The 'nervous system' — handles communication between all software parts |
| --- | --- |
| Gazebo Harmonic | The 3D physics simulator — test the arm safely before touching real hardware |
| URDF / Xacro | The 'blueprint' files that describe the arm's shape and joints in XML |
| ros2_control | The 'motor driver' — sends joint angle commands to real or simulated motors |
| OpenCV | Basic computer vision — detect colours, shapes, contours from camera images |
| YOLOv8 | Advanced AI — identifies objects by what they look like, not just their colour |
| MediaPipe | Google's hand-tracking library — finds 21 keypoints on your hand per frame |
| MQTT | A lightweight messaging protocol — the 'WhatsApp for machines' over the internet |
| Python | The main programming language for all vision, gesture, and control logic |
| Arduino + C++ | Controls the physical servo motors when the arm is real hardware |

## 0.3  How the Pieces Connect

Picture it like a chain:

- Camera sees the world → OpenCV/YOLOv8/MediaPipe understands what it sees → ROS2 nodes share that information → Mode Manager decides what to do → ros2_control sends commands to joints → Gazebo (or real Arduino) moves the arm.

Every technology in this list serves exactly one purpose in that chain. Once you understand each link, the whole system makes perfect sense.

# SECTION 1 — Robotic Arm Fundamentals

## 1.1  What Is a Robotic Arm?

A robotic arm is a mechanical structure that mimics the range of motion of a human arm. It is made of rigid segments called 'links' connected by movable joints. By rotating each joint by a specific angle, the arm can reach any point in 3D space.

Think of your own arm: your shoulder rotates in multiple directions, your elbow bends, your wrist twists. A robotic arm works the same way — each joint is a motor that rotates when given a command.

## 1.2  Degrees of Freedom (DOF)

'Degree of Freedom' (DOF) means: in how many independent ways can a joint or the whole arm move?

- 1 DOF = moves in exactly one direction (e.g., a door hinge — it only rotates around one axis)

- 6 DOF = your arm can reach any position in space AND rotate itself to any angle. This is the magic number for industrial arms.

Your project is a 6-DOF arm. Why 6? Because to fully position an object in 3D space — its X, Y, Z position AND its pitch, roll, yaw rotation — you need exactly 6 independent movements.

💡 Analogy: A camera on a gimbal has 3 DOF — it can pan left/right, tilt up/down, and roll. Add 3 more axes and you can control a full robotic arm.

## 1.3  Types of Joints

Your arm uses two main types of joints:

- Revolute Joint — Rotates around a fixed axis, like a hinge. Most robot joints are this type. Controlled by giving it an angle in radians.

- Prismatic Joint — Slides along a linear axis, like a drawer. Used in the gripper to open and close.

## 1.4  Forward Kinematics vs Inverse Kinematics

These are two fundamental ways of thinking about how a robot arm moves:

- Forward Kinematics (FK) — You KNOW the angles of every joint and you CALCULATE where the end of the arm (gripper) ends up in space. Easy maths, deterministic.

- Inverse Kinematics (IK) — You KNOW where you WANT the gripper to go and you CALCULATE what angles every joint needs to be at. This is hard — there are often multiple solutions or no solution.

💡 MoveIt2 handles Inverse Kinematics for you automatically. You tell it 'go to position X,Y,Z' and it figures out the joint angles. This is one of its most valuable features.

## 1.5  The Gripper (6th DOF)

Your base model has 5 DOF. You are adding a 6th — the gripper. A gripper is the 'hand' at the end of the arm. In your project, it uses two fingers driven by a prismatic joint. When the joint value is 0, the fingers are closed. As the value increases, they open.

The gripper is what actually picks up objects. The entire arm positions itself, then the gripper is commanded to close around the object.

# SECTION 2 — ROS2 (Robot Operating System 2)

## 2.1  What Is ROS2? Why Does It Exist?

ROS2 (Robot Operating System 2) is NOT an operating system like Windows or Linux. It is a middleware framework — a collection of tools, libraries, and conventions that helps different pieces of robot software talk to each other.

Imagine you are building a robot with 10 different programs: one reads the camera, one controls motors, one does AI detection, one handles user input, etc. Without ROS2, you'd have to manually wire all these programs together, handle networking, timing, and error recovery yourself. ROS2 does all of that for you.

💡 Analogy: ROS2 is like the postal system of your robot. Each software 'node' is a building. ROS2 is the postal network that delivers messages between them. You don't need to know HOW the letter gets there — you just address it and drop it in the mailbox.

## 2.2  The Core Concepts

### Nodes

A node is simply one standalone program in ROS2. Your project will have many nodes running at the same time:

- camera_node — reads frames from the camera

- vision_node — runs OpenCV/YOLO on those frames

- gesture_node — runs MediaPipe on the frames

- mode_manager_node — decides which mode the robot is in

- arm_controller_node — sends joint commands to the arm

Each node runs independently. If one crashes, the others keep running.

### Topics

Nodes communicate by publishing messages to 'topics' and subscribing to topics. A topic is like a named radio channel.

- The camera_node PUBLISHES images to the topic /camera/image_raw

- The vision_node SUBSCRIBES to /camera/image_raw and receives every frame

- The vision_node then PUBLISHES results to /detected_objects

- The mode_manager_node SUBSCRIBES to /detected_objects

This is called the pub-sub (publish-subscribe) pattern. Publishers don't know who reads their messages and subscribers don't know who sends them. This makes the system modular and flexible.

### Messages

Messages are the data packets sent over topics. ROS2 has standard message types for common things:

- sensor_msgs/Image — A camera frame (raw pixel data)

- geometry_msgs/Pose — A 3D position and orientation

- std_msgs/String — A simple text string

- trajectory_msgs/JointTrajectory — A sequence of joint angle commands

You can also define your own custom message types for your project's specific data.

### Services

Topics are one-to-many and continuous (like a TV broadcast). Services are one-to-one and request-response (like a phone call). You use a service when you need a direct answer back. Example: 'Is the arm currently in its home position? Yes/No.'

### Actions

Actions are like services but for long-running tasks. You request an action (e.g., 'move arm to position X'), and the action server keeps sending you progress updates until it's done or fails. MoveIt2 uses actions for trajectory execution.

## 2.3  The ROS2 Workspace

All your code lives in a 'workspace' — a folder with a specific structure that ROS2 understands:

ros2_ws/

src/              ← All your ROS2 packages (your actual code)

build/            ← Compiled output (auto-generated, don't touch)

install/          ← Installed packages (auto-generated)

log/              ← Build and run logs

Inside src/, each 'package' is a self-contained module with its own code, dependencies, and configuration. Your project has multiple packages like arm_description, arm_vision, arm_controller, etc.

## 2.4  Building and Running

You build your workspace with one command:

colcon build

Then you 'source' it so your terminal knows where to find your packages:

source install/setup.bash

Then you run a node:

ros2 run my_package my_node

Or launch an entire system at once with a launch file:

ros2 launch my_package my_launch.py

💡 A launch file is a Python script that starts multiple nodes at once with their configurations. You'll have one launch file that starts Gazebo, the arm, the vision system, and the controller all at once.

## 2.5  ROS2 Jazzy — Why This Version?

'Jazzy' is the name of the specific ROS2 release (like Ubuntu 24.04 has a codename). Jazzy runs on Ubuntu 24.04 and is the latest LTS (Long Term Support) version. It works with Gazebo Harmonic. You must use matching versions — Jazzy + Harmonic — or things will break.

# SECTION 3 — Gazebo Harmonic (The Simulator)

## 3.1  What Is Gazebo?

Gazebo is a 3D physics simulator. It creates a virtual world with gravity, collisions, friction, and light — just like the real world — where you can test your robot without risk of breaking anything.

Think of it as a very advanced video game engine but for robots. You can place your robotic arm in a virtual room, put virtual objects on a virtual table, and run your actual control code against it. When it works in Gazebo, it'll (mostly) work in real life.

💡 Why simulate first? Real robot arms cost hundreds of dollars and can break. A programming bug that sends a joint past its limit might snap a servo. In Gazebo, you can crash and restart infinitely at zero cost.

## 3.2  Gazebo Harmonic — The New Architecture

'Harmonic' is the latest generation of Gazebo (now called Gazebo Sim, previously Ignition Gazebo). It has a significantly different architecture from the older 'Gazebo Classic':

- Plugin-based — Everything in Gazebo (sensors, physics, rendering) is a plugin. This makes it modular and powerful.

- Entity Component System (ECS) — Instead of object hierarchies, Gazebo uses 'entities' (things in the world) with attached 'components' (data about those things). This is a modern architecture used in game engines too.

- ros_gz bridge — The connection between ROS2 and Gazebo is a 'bridge' that translates ROS2 topics to Gazebo topics and vice versa.

## 3.3  How Your Arm Gets Into Gazebo

The process is:

- You write a URDF/Xacro file (the arm's blueprint — covered in Section 4)

- Gazebo reads that file and creates a 3D model of your arm

- ros2_control plugins in Gazebo intercept joint commands from ROS2 and move the simulated arm

- Simulated camera sensors publish images as ROS2 topics — your vision code reads those

From your code's perspective, it doesn't matter if it's talking to a simulated arm or a real one — the ROS2 topics look exactly the same.

## 3.4  Key Gazebo Concepts

- World file (.sdf) — Describes the entire simulation environment: gravity, lighting, ground plane, objects, and the robot. SDF is similar to URDF but richer.

- Model — Any physical object in the simulation (your arm, the table, the objects to sort).

- Plugin — A C++ library that adds behaviour to a model or sensor. ros2_control uses a Gazebo plugin to intercept joint commands.

- gz topic — Gazebo has its own internal topic system (separate from ROS2 topics). The ros_gz bridge connects the two.

# SECTION 4 — URDF & Xacro (The Robot Blueprint)

## 4.1  What Is URDF?

URDF stands for Unified Robot Description Format. It is an XML file that completely describes your robot:

- The physical shape and size of each link (segment)

- The type and axis of each joint (revolute, prismatic, fixed)

- The limits of each joint (min/max angle, max speed, max torque)

- The mass and inertia of each link (needed for physics simulation)

- The visual appearance (3D mesh files) and collision geometry

- Sensor positions (where cameras are mounted)

Think of it as the CAD blueprint for your robot, but in a text format that ROS2 and Gazebo can read programmatically.

## 4.2  URDF Structure — The Key Tags

A URDF file consists of just a few key elements:

<robot name='my_arm'>

<link name='base_link'>          ← A rigid segment

<visual> ... </visual>          ← What it looks like

<collision> ... </collision>    ← For collision detection

<inertial> ... </inertial>      ← Mass + moment of inertia

</link>

<joint name='joint1' type='revolute'>   ← A revolute (rotating) joint

<parent link='base_link'/>             ← What it connects FROM

<child link='link1'/>                  ← What it connects TO

<origin xyz='0 0 0.1' rpy='0 0 0'/>   ← Position & rotation offset

<axis xyz='0 0 1'/>                    ← Rotation axis (Z-axis here)

<limit lower='-3.14' upper='3.14' effort='100' velocity='1'/>

</joint>

</robot>

The robot is built as a tree of links connected by joints, starting from base_link.

## 4.3  What Is Xacro?

URDF files get very repetitive and long. Xacro (XML Macros) is a preprocessor that adds variables and macros to URDF, making it much shorter and easier to maintain.

- Variables — Instead of repeating '0.05' everywhere, define a variable: <xacro:property name='link_radius' value='0.05'/>

- Macros — Define a template once and reuse it for similar joints: <xacro:macro name='revolute_joint' params='name parent child'>

- Include files — Split your robot into multiple files: <xacro:include filename='gripper.xacro'/>

Your project starts from an existing URDF (ferasboulala's 5-DOF arm) and adapts it. You'll mainly be reading and understanding the URDF, adding the gripper joint, and adding ros2_control tags.

## 4.4  Adding ros2_control Tags to URDF

To make ros2_control work, you add special tags inside your URDF that tell ROS2 which joints are controllable:

<ros2_control name='GazeboSystem' type='system'>

<hardware>

<plugin>gz_ros2_control/GazeboSimSystem</plugin>

</hardware>

<joint name='joint1'>

<command_interface name='position'/>   ← We send position commands

<state_interface name='position'/>     ← We can read current position

<state_interface name='velocity'/>

</joint>

... (repeat for all 6 joints)

</ros2_control>

# SECTION 5 — ros2_control (The Motor Controller)

## 5.1  What Is ros2_control?

ros2_control is the ROS2 framework for robot hardware control. It sits between your high-level motion planning (MoveIt2) and the actual hardware (motors/servos or Gazebo simulation). It provides a clean, standardised way to control any robot hardware.

💡 Analogy: ros2_control is like a universal remote control. MoveIt2 is the person using it. The TV (or Gazebo, or real servos) is the hardware. The universal remote translates the 'channel up' button into whatever signal the specific TV needs — you don't need to know the TV's internal workings.

## 5.2  Key Components

- Hardware Interface — A plugin that knows HOW to talk to a specific piece of hardware (Gazebo simulation or real servo). Your URDF specifies which plugin to use.

- Controller Manager — The central node that loads and manages all controllers. It runs as a ROS2 node.

- Controllers — Specific control algorithms. You'll use two:

- joint_trajectory_controller — Accepts a trajectory (sequence of joint angles over time) and executes it smoothly. This is what MoveIt2 sends commands to.

- joint_state_broadcaster — Reads the current joint positions and publishes them as /joint_states. This is how RViz and other nodes know where the arm currently is.

## 5.3  The Control Loop

ros2_control runs a tight loop (typically at 100Hz — 100 times per second):

- READ: Query the hardware for current joint positions/velocities

- UPDATE: Run the controller algorithm (compute the next command based on current state and target)

- WRITE: Send the command to the hardware

This loop is deterministic and real-time. It's what makes the arm move smoothly rather than in jerky steps.

## 5.4  Configuration Files

ros2_control is configured via YAML files:

controller_manager:

ros__parameters:

update_rate: 100  # Hz

joint_trajectory_controller:

ros__parameters:

joints: [joint1, joint2, joint3, joint4, joint5, joint6]

command_interfaces: [position]

state_interfaces: [position, velocity]

# SECTION 6 — MoveIt2 (Motion Planning)

## 6.1  What Is MoveIt2?

MoveIt2 is the motion planning framework for ROS2. It takes a goal (e.g., 'move the gripper to position X=0.3, Y=0.1, Z=0.5') and calculates a safe, collision-free path to get there, then executes it via ros2_control.

MoveIt2 handles some of the hardest problems in robotics:

- Inverse Kinematics — Given a target position in 3D space, compute the required joint angles

- Collision avoidance — Ensure the arm doesn't hit itself or the environment

- Trajectory planning — Smooth out the motion so joints accelerate and decelerate gracefully

- Workspace limits — Respect joint limits defined in the URDF

## 6.2  How You Use MoveIt2

From your Python code, MoveIt2 is very straightforward:

move_group = MoveGroupInterface(node, 'arm')

move_group.set_position_target([0.3, 0.1, 0.5])

move_group.move()   ← MoveIt2 handles EVERYTHING else

The MoveGroup interface is your main tool. You set a target, call move(), and MoveIt2 calculates the path, checks for collisions, and sends the trajectory to ros2_control.

# SECTION 7 — Computer Vision with OpenCV

## 7.1  What Is OpenCV?

OpenCV (Open Source Computer Vision Library) is the industry-standard Python/C++ library for image processing. It can read camera images, manipulate pixels, detect colours, find edges, and much more — all in real time.

In your project, OpenCV is used for fast colour-based object detection. It's simpler and faster than AI (YOLOv8) and runs at 30+ fps easily.

## 7.2  How a Camera Image Is Stored

To a computer, an image is just a 3D array of numbers (a NumPy array):

- Height × Width × 3 channels (for a colour image)

- Each pixel has 3 values: Blue, Green, Red (in OpenCV's BGR order, not RGB!)

- Each value ranges from 0 to 255

img = cv2.imread('photo.jpg')   # shape: (480, 640, 3)

img[240, 320]  # → [B, G, R] values of pixel at row 240, column 320

## 7.3  Colour Detection — The HSV Approach

To detect a red object, you can't just look for pixels where Red=255, Green=0, Blue=0. Real-world colours vary with lighting. The HSV colour space solves this:

- H — Hue — The actual colour (0-179 in OpenCV). Red is around 0 or 170. Green is ~60. Blue is ~120.

- S — Saturation — How 'vivid' the colour is (0=grey, 255=pure colour).

- V — Value — How bright the colour is (0=black, 255=fully lit).

To detect red objects: convert image to HSV, create a 'mask' that keeps only pixels in a red hue range, then find where that mask has bright connected regions.

hsv = cv2.cvtColor(img, cv2.COLOR_BGR2HSV)

lower_red = np.array([0, 100, 100])    # Min H, S, V for red

upper_red = np.array([10, 255, 255])   # Max H, S, V for red

mask = cv2.inRange(hsv, lower_red, upper_red)  # White=red, Black=not-red

## 7.4  Finding Object Location — Contours

After creating a mask, you find the boundaries of white regions (contours):

contours, _ = cv2.findContours(mask, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)

for cnt in contours:

area = cv2.contourArea(cnt)          # Filter small noise

x, y, w, h = cv2.boundingRect(cnt)  # Bounding box of the object

cx, cy = x + w//2, y + h//2         # Centre pixel of the object

cx, cy gives you the pixel coordinates of the object. The next step (camera calibration) converts these pixel coordinates to real-world centimetres.

## 7.5  ROS2 + OpenCV Integration

Camera images arrive as ROS2 messages (sensor_msgs/Image). You use a library called cv_bridge to convert between ROS2 image messages and OpenCV NumPy arrays:

from cv_bridge import CvBridge

bridge = CvBridge()

cv_image = bridge.imgmsg_to_cv2(ros_msg, 'bgr8')  # ROS → OpenCV

ros_msg = bridge.cv2_to_imgmsg(cv_image, 'bgr8')  # OpenCV → ROS

# SECTION 8 — Camera Calibration & Coordinate Conversion

## 8.1  The Problem: Pixels Are Not Centimetres

Your vision code tells you an object is at pixel (320, 240). But the arm needs to know: how many centimetres away is the object? And in which direction?

A camera image is a 2D projection of a 3D world. Converting from 2D pixels back to 3D real-world coordinates requires knowing the camera's intrinsic properties.

## 8.2  Camera Intrinsics — The 3 Things You Need

- Focal length (fx, fy) — How 'zoomed in' the lens is. Measured in pixels.

- Principal point (cx, cy) — The pixel coordinate of the centre of the image (usually near the image centre, but not exactly).

- Distortion coefficients — Real lenses are not perfect — they introduce barrel or pincushion distortion that makes straight lines look curved. These 5 numbers describe the distortion.

These values together form the 'camera matrix' K. Once you know K, you can mathematically un-project any pixel back into 3D space.

## 8.3  How to Calibrate Your Camera

Camera calibration is done once, using a chessboard pattern:

- Print or display a chessboard pattern (e.g., 9x6 inner corners)

- Take 15-20 photos of it from different angles

- Run OpenCV's calibrateCamera() function on those photos

- It outputs your camera matrix K and distortion coefficients D

- Save these to a YAML file and load it in your vision nodes

ret, K, D, rvecs, tvecs = cv2.calibrateCamera(objpoints, imgpoints, img_size, None, None)

💡 You only do this calibration once. The camera matrix stays constant for a given camera unless you change its lens or resolution.

## 8.4  Pixel to Real-World Conversion

Once calibrated, converting a 2D pixel to a 3D point requires knowing the depth (distance) to the object. You have two options:

- Known depth method — If your conveyor is always at a fixed height (e.g., 30cm below the camera), you know the depth Z. Then: X_real = (pixel_x - cx) * Z / fx

- Depth camera — An RGBD camera (like Intel RealSense) directly gives you depth for every pixel, making 3D reconstruction trivial.

For your project, the simpler known-depth method works fine because the sorting conveyor is at a fixed, known height.

# SECTION 9 — YOLOv8 (AI Object Detection)

## 9.1  What Is YOLOv8?

YOLO stands for 'You Only Look Once'. It's a family of deep learning models that can detect and classify objects in an image in real time. YOLOv8 is the latest version by Ultralytics.

Unlike OpenCV colour detection (which only knows colour), YOLO can recognise what an object IS — a cube vs a cylinder vs a bottle — based on its shape and texture. It's trained on millions of images and can identify 80 common objects out of the box.

## 9.2  How YOLO Works (Conceptually)

You don't need to understand the deep learning math. At a high level:

- The image is divided into a grid

- Each grid cell predicts: 'Is there an object here? If so, what is it? And where exactly?'

- A neural network (a mathematical function with millions of learned parameters) processes the entire image in one pass — hence 'You Only Look Once'

- The output is a list of bounding boxes: [x, y, width, height, class_name, confidence_score]

- Confidence score tells you how sure the model is (0.0 to 1.0)

## 9.3  Using YOLOv8 in Python

Ultralytics makes YOLOv8 remarkably simple to use:

from ultralytics import YOLO

model = YOLO('yolov8n.pt')   # 'n' = nano (smallest, fastest)

results = model(image_frame)  # Run detection

for r in results:

for box in r.boxes:

class_name = model.names[int(box.cls)]  # e.g., 'bottle'

x1,y1,x2,y2 = box.xyxy[0]               # Bounding box coords

confidence = float(box.conf)             # e.g., 0.92

## 9.4  CUDA Acceleration

YOLO on a CPU runs at ~5 fps — too slow for real-time sorting. On an NVIDIA GPU with CUDA, it runs at 60+ fps. CUDA is NVIDIA's framework for running computation on the GPU.

When you install PyTorch with CUDA support, YOLOv8 automatically uses the GPU:

model = YOLO('yolov8n.pt').cuda()   # Force GPU usage

Your development machine needs an NVIDIA GPU and CUDA installed for this.

## 9.5  Custom Training (If Needed)

The default YOLO model recognises 80 common objects. If you're sorting custom objects (e.g., specific coloured cubes), you need to:

- Collect ~100+ images of your objects

- Label them using Roboflow (draw bounding boxes and assign class names)

- Export the dataset in YOLO format

- Fine-tune the model: model.train(data='dataset.yaml', epochs=50)

For your project's objects (cubes, cylinders by colour), OpenCV colour detection may be sufficient and simpler.

# SECTION 10 — MediaPipe Hand Gesture Recognition

## 10.1  What Is MediaPipe?

MediaPipe is Google's open-source framework for perceptual processing pipelines. For your project, you'll use its Hand Landmarker solution, which detects and tracks hands in real time.

## 10.2  How Hand Tracking Works

MediaPipe's hand model detects 21 key points (called 'landmarks') on each hand in every frame:

- Wrist (1 point)

- Each finger has 4 points: MCP (knuckle), PIP, DIP, TIP (fingertip)

- That's 4 fingers × 4 points + 3 thumb points + 1 wrist = 21 points total

Each landmark has X, Y (normalised 0-1 coordinates in the image) and Z (depth estimate). From these 21 points, you can compute any gesture.

## 10.3  Gesture-to-Robot Mapping

Your project maps hand position and orientation to robot arm joint angles:

- Hand Y position — Maps to shoulder joint angle (arm height)

- Hand X position — Maps to rotation around the base

- Wrist roll — Maps to the arm's wrist rotation joint

- Finger pinch distance — Controls the gripper open/close

The maths is just linear mapping: a hand at 10% from the top → shoulder at maximum angle; hand at 90% from the top → shoulder at minimum angle.

def map_range(value, in_min, in_max, out_min, out_max):

return (value - in_min) / (in_max - in_min) * (out_max - out_min) + out_min

hand_y = landmarks[0].y           # Wrist Y position (0.0 to 1.0)

shoulder_angle = map_range(hand_y, 0.0, 1.0, -1.57, 1.57)  # -90° to +90°

## 10.4  Using MediaPipe in Python

import mediapipe as mp

mp_hands = mp.solutions.hands

hands = mp_hands.Hands(max_num_hands=1, min_detection_confidence=0.7)

frame_rgb = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)

results = hands.process(frame_rgb)

if results.multi_hand_landmarks:

landmarks = results.multi_hand_landmarks[0].landmark

wrist_x = landmarks[0].x   # Wrist X (0.0 = left edge, 1.0 = right edge)

wrist_y = landmarks[0].y   # Wrist Y (0.0 = top, 1.0 = bottom)

# SECTION 11 — MQTT (Remote Control Protocol)

## 11.1  What Is MQTT?

MQTT (Message Queuing Telemetry Transport) is a lightweight messaging protocol designed for IoT (Internet of Things) devices. It's the 'WhatsApp for machines' — designed to send small messages between devices over the internet with very low bandwidth.

💡 Analogy: MQTT works exactly like ROS2 topics but over the internet. You have a 'broker' (server) in the middle, publishers send messages to topics on the broker, and subscribers receive them — potentially from anywhere in the world.

## 11.2  The MQTT Architecture

- Broker — A server that receives all messages and forwards them to the right subscribers. You'll use HiveMQ Cloud (free tier) as your broker.

- Publisher — Your phone's browser, running MediaPipe JS, publishes hand landmark data to a topic like 'robot/hand/landmarks' every frame.

- Subscriber — Your robot computer subscribes to that same topic and receives the data.

The phone and robot computer don't connect to each other directly — they both connect to the HiveMQ broker in the cloud.

## 11.3  MQTT Topics

Like ROS2 topics, MQTT topics are just named channels. You'll define:

robot/hand/landmarks    ← Phone publishes hand position data here

robot/mode/current      ← Robot publishes its current mode here

robot/mode/request      ← Remote user can request a mode change

## 11.4  MQTT in Python

import paho.mqtt.client as mqtt

client = mqtt.Client()

client.connect('broker.hivemq.com', 1883)

# Subscribe

def on_message(client, userdata, msg):

data = json.loads(msg.payload)  # Parse JSON

process_hand_landmarks(data)

client.subscribe('robot/hand/landmarks')

client.on_message = on_message

client.loop_start()   # Background thread for incoming messages

## 11.5  The Phone Side — MediaPipe JS + MQTT

The phone runs a simple HTML page in its browser that:

- Accesses the phone camera via getUserMedia() API

- Runs MediaPipe JS (a JavaScript version of MediaPipe) on each frame

- Publishes the 21 hand landmarks as JSON over MQTT WebSocket

No app installation needed — just a web page. This is why ngrok is used — it creates a temporary public URL so the phone can reach your robot computer if needed.

# SECTION 12 — Python OOP (Object-Oriented Programming)

## 12.1  Why OOP Matters for This Project

Every ROS2 node is written as a Python class. When you run a node, Python creates an instance of that class. Understanding classes and how they work is non-negotiable for this project.

## 12.2  Classes and Objects

A class is a blueprint. An object is a specific instance created from that blueprint:

class Dog:

def __init__(self, name, breed):  # Constructor — runs on creation

self.name = name               # Instance variable

self.breed = breed

def bark(self):                   # Method (function belonging to the class)

print(f'{self.name} says Woof!')

my_dog = Dog('Rex', 'Labrador')   # Create an object

my_dog.bark()                     # Output: Rex says Woof!

'self' refers to the specific instance. Every method must have 'self' as its first parameter.

## 12.3  A ROS2 Node as a Class

In ROS2, every node is a class that inherits from rclpy.node.Node:

import rclpy

from rclpy.node import Node

from sensor_msgs.msg import Image

class VisionNode(Node):

def __init__(self):

super().__init__('vision_node')   # Name of the node

self.sub = self.create_subscription(

Image,                          # Message type

'/camera/image_raw',            # Topic name

self.image_callback,            # Function to call

10                              # Queue size

)

def image_callback(self, msg):

# Called AUTOMATICALLY every time a new image arrives

self.get_logger().info('Got an image!')

def main():

rclpy.init()

node = VisionNode()

rclpy.spin(node)   # Keep node alive, process callbacks

if __name__ == '__main__': main()

## 12.4  Callbacks — The Most Important Concept

A callback is a function that you DON'T call directly — the framework calls it for you when something happens. In ROS2, callbacks are called when:

- Subscription callback — A new message arrives on a subscribed topic

- Timer callback — A timer fires (e.g., every 100ms)

- Service callback — A service request arrives

rclpy.spin(node) keeps your node alive and processes these callbacks as they arrive. Think of it as the event loop.

## 12.5  NumPy Basics

NumPy is Python's library for fast numerical arrays. Camera images in OpenCV ARE NumPy arrays. You need to know:

import numpy as np

arr = np.array([1, 2, 3, 4, 5])

arr * 2          # → [2, 4, 6, 8, 10] — element-wise operation

arr[arr > 3]     # → [4, 5] — boolean indexing

np.mean(arr)     # → 3.0

img = np.zeros((480, 640, 3), dtype=np.uint8)  # Black image

img[100:200, 150:250] = [0, 255, 0]            # Green rectangle

# SECTION 13 — Arduino & Real Hardware

## 13.1  What Is Arduino?

Arduino is an open-source microcontroller platform. A microcontroller is a tiny computer on a single chip — it has a processor, memory, and input/output pins, but no operating system. You program it in C++ to do one specific task repeatedly.

In your project, the Arduino receives joint angle commands from ROS2 (via serial USB) and translates them into PWM signals that drive the servo motors.

## 13.2  Servo Motors

A servo motor is different from a regular motor:

- Regular motor — Spins continuously when power is applied

- Servo motor — Moves to a specific angle when commanded and holds it

Servos receive a PWM (Pulse Width Modulation) signal. The width of each pulse determines the target angle. A 1ms pulse = 0°, 1.5ms pulse = 90°, 2ms pulse = 180°.

## 13.3  PCA9685 — The Servo Driver

A standard Arduino has only a few PWM-capable pins. Your arm has 6 joints. The PCA9685 is a dedicated PWM driver chip (I2C interface) that can control up to 16 servos. The Arduino talks to it via the I2C protocol (two wires: SDA and SCL).

// Arduino C++ code

#include <Adafruit_PWMServoDriver.h>

Adafruit_PWMServoDriver pwm = Adafruit_PWMServoDriver();

void setAngle(int channel, float angle_degrees) {

int pulse = map(angle_degrees, 0, 180, SERVOMIN, SERVOMAX);

pwm.setPWM(channel, 0, pulse);

}

## 13.4  ROS2 to Arduino Communication

Your Arduino firmware reads joint angle commands from the USB serial port. The ROS2 hardware interface plugin writes to that serial port:

- ROS2 hardware interface sends: 'J1:45.0,J2:30.0,J3:90.0\n' via serial

- Arduino parses that string, extracts the 6 angles, calls setAngle() for each

- Arduino sends back current positions: 'J1:45.0,J2:30.1,...\n' for feedback

This is the 'Hardware Interface' layer of ros2_control — you write it in C++ following the ros2_control API.

## 13.5  The Transition from Simulation to Hardware

This is one of the cleverest things about ros2_control. Your simulation uses the GazeboSimSystem plugin. Your real hardware uses your custom SerialServoHardware plugin. But from MoveIt2's perspective, NOTHING changes — it just calls the same ros2_control API. You switch hardware by changing one line in your URDF config:

<!-- Simulation -->

<plugin>gz_ros2_control/GazeboSimSystem</plugin>

<!-- Real hardware -->

<plugin>arm_hardware/SerialServoHardware</plugin>

# SECTION 14 — The Mode Manager (The Brain)

## 14.1  What Is the Mode Manager?

The Mode Manager is a central ROS2 node that watches all inputs and decides which operational mode the arm should be in. It's the decision-making hub of the system.

At any moment, the system is in exactly ONE of three modes:

- AUTONOMOUS — No hand detected in camera view. Vision pipeline runs. Arm sorts objects.

- LOCAL_GESTURE — A hand is detected in the local webcam. Hand movements control the arm.

- REMOTE_GESTURE — MQTT messages are arriving from the phone. Phone hand controls the arm.

## 14.2  Mode Switching Logic

The mode manager uses a simple priority system:

- Check MQTT — if hand data is arriving from MQTT, enter REMOTE_GESTURE mode

- Else check local camera — if MediaPipe detects a hand, enter LOCAL_GESTURE mode

- Else — remain in AUTONOMOUS mode

Safety timeouts: if MQTT data stops for 2 seconds (phone connection lost), automatically fall back to LOCAL_GESTURE or AUTONOMOUS. The arm also returns to its 'home' position on mode change to avoid erratic movement.

## 14.3  Implementing the State Machine

This is a simple finite state machine — a very common programming pattern:

from enum import Enum

class Mode(Enum):

AUTONOMOUS = 'autonomous'

LOCAL_GESTURE = 'local_gesture'

REMOTE_GESTURE = 'remote_gesture'

class ModeManager(Node):

def __init__(self):

super().__init__('mode_manager')

self.current_mode = Mode.AUTONOMOUS

self.last_mqtt_time = None

self.hand_detected_locally = False

self.create_timer(0.1, self.update_mode)  # Check every 100ms

def update_mode(self):

mqtt_active = (time.time() - self.last_mqtt_time) < 2.0

if mqtt_active:

self.switch_to(Mode.REMOTE_GESTURE)

elif self.hand_detected_locally:

self.switch_to(Mode.LOCAL_GESTURE)

else:

self.switch_to(Mode.AUTONOMOUS)

# SECTION 15 — Your Week-by-Week Learning Roadmap

Based on your project's Learning Path (Section 10.2 in the documentation), here is a practical guide for each week — what to do, what to build, and how to know you've succeeded.

## Week 1 — Python OOP

Goal: Be comfortable writing ROS2 node classes.

- Watch — Corey Schafer's Python OOP playlist on YouTube (free)

- Practice — Write a simple class. Write a class with a timer callback. Write two classes that talk to each other via a list.

- Success metric — You can write a ROS2 node class from memory: __init__, create_subscription, create_timer, callback methods, rclpy.spin()

- NumPy — Go through Keith Galli's NumPy crash course. Practice array slicing and broadcasting.

## Week 2 — OpenCV Basics

Goal: Write a colour detector that draws a bounding box around red objects.

- Install — pip install opencv-python

- Practice — Open your webcam with cv2.VideoCapture(0). Convert each frame to HSV. Create a colour mask. Find contours. Draw a rectangle.

- Success metric — You have a Python script that opens your webcam and draws a green box around any red object in real time.

- Integrate with ROS2 — Wrap this in a ROS2 node that subscribes to /camera/image_raw

## Week 3 — Camera Calibration

Goal: Get a calibrated camera matrix. Know how to convert pixels to real-world coordinates.

- Print — A 9x6 chessboard calibration pattern from the internet

- Collect — 20 images of the chessboard from different angles using your webcam

- Run — OpenCV's calibrateCamera() function

- Success metric — You have a camera_matrix.yaml file with your fx, fy, cx, cy, and distortion coefficients. You can use them to undistort an image.

## Week 4 — YOLOv8

Goal: Run YOLOv8 on your webcam and see bounding boxes around objects.

- Install — pip install ultralytics

- Run — The 3-line YOLO demo from Section 9.3 above on your webcam

- Integrate — Wrap it in a ROS2 node. Subscribe to camera images. Publish detected object locations as ROS2 messages.

- Success metric — Your ROS2 node publishes the class and location of objects detected by the camera

## Week 5 — MediaPipe Gestures

Goal: Map your hand position to 6 joint angle values.

- Install — pip install mediapipe

- Practice — Display the 21 hand landmarks on your webcam feed. Print out landmark 0 (wrist) X,Y values.

- Implement — The map_range function from Section 10.3. Map wrist position to 6 joint angles.

- Success metric — Moving your hand in front of your webcam produces changing joint angle values that a ROS2 topic publishes

## Week 6 — MQTT Remote Control

Goal: Your phone controls the robot from another room (or another country).

- Set up — A free HiveMQ Cloud account. Get your broker URL and credentials.

- Test — Write a Python MQTT publisher and subscriber. Publisher sends 'hello' every second. Subscriber prints it.

- Build — The HTML/JS phone page with MediaPipe JS that publishes hand data over MQTT

- Success metric — You can see the hand data from your phone arriving in a Python subscriber on your robot computer

## Hardware Phase — Arduino & Real Arm

This phase begins after the simulation is complete.

- Write — The Arduino firmware that reads serial commands and drives servos via PCA9685

- Test — The Arduino independently by sending manual serial commands from Arduino IDE

- Write — The ros2_control hardware interface plugin in C++ that talks to the Arduino over serial

- Success metric — Switching the URDF plugin from GazeboSimSystem to your hardware plugin makes the physical arm move the same way the simulated arm did

# SECTION 16 — Quick Reference Glossary

Every technical term you'll encounter, explained in one sentence:

| 6-DOF | A robot with 6 independent joints — can reach any position and orientation in 3D space |
| --- | --- |
| Autonomous Mode | Robot sorts objects by itself using computer vision, no human needed |
| Bounding Box | A rectangle drawn around a detected object in an image (x, y, width, height) |
| Broker (MQTT) | A server in the cloud that routes messages between publishers and subscribers |
| Callback | A function the framework calls automatically when an event occurs (new message, timer tick) |
| Camera Matrix K | 3x3 matrix containing focal lengths and principal point — needed for 3D reconstruction |
| Class (Python) | A blueprint for creating objects with shared properties and methods |
| colcon build | The command to compile your entire ROS2 workspace |
| Contour | The boundary outline of a region in a binary image (found by cv2.findContours) |
| Controller Manager | ROS2 node that loads and runs hardware controllers |
| CUDA | NVIDIA's framework for running parallel computations on a GPU — makes AI much faster |
| Depth (Z) | The distance from the camera to an object — needed to convert 2D pixels to 3D coords |
| DOF | Degree of Freedom — number of independent directions a joint or whole robot can move |
| Distortion | Lens imperfection that makes straight lines appear curved in images |
| Forward Kinematics | Given joint angles → calculate where the arm tip ends up |
| Gazebo Harmonic | The 3D physics simulator for testing robots without real hardware |
| Gesture Mode | Human hand movements control the robot arm via camera tracking |
| Gripper | The 'hand' at the end of the arm — opens and closes to pick up objects |
| HSV | Hue-Saturation-Value colour space — better than RGB for colour detection under varying lighting |
| Inverse Kinematics | Given a target 3D position → calculate the required joint angles |
| I2C | A two-wire serial communication protocol used to control the PCA9685 servo driver |
| Joint | The movable connection between two rigid links in a robot arm |
| Joint State | The current angle/position of all joints — published to /joint_states topic |
| Launch file | A Python script that starts multiple ROS2 nodes simultaneously with their configs |
| Link | A rigid segment of the robot arm between two joints |
| MediaPipe | Google's hand/face/pose tracking library — gives 21 hand landmark positions per frame |
| Mode Manager | The central ROS2 node that decides which operational mode the robot is in |
| MoveIt2 | ROS2's motion planning framework — handles IK, collision avoidance, trajectory planning |
| MQTT | Lightweight IoT messaging protocol — used for phone-to-robot internet communication |
| Node (ROS2) | A single running program that is part of the ROS2 system |
| NumPy | Python library for fast numerical arrays — camera images are stored as NumPy arrays |
| OpenCV | The standard library for image processing — colour detection, contours, calibration |
| PCA9685 | 16-channel PWM servo driver chip, controlled via I2C, drives up to 16 servos |
| Principal Point | The pixel at the exact optical centre of the camera (usually near the image centre) |
| pub-sub | Publish-Subscribe pattern — publishers send to topics, subscribers receive from topics |
| PWM | Pulse Width Modulation — how servos are controlled: pulse width determines target angle |
| rclpy | The Python client library for ROS2 — you import this to create nodes |
| Revolute Joint | A joint that rotates around an axis, like a hinge — most robot joints are this type |
| ros2_control | The ROS2 framework for standardised robot hardware control |
| ros_gz bridge | The ROS2-Gazebo communication bridge — translates between their different topic systems |
| Serial (UART) | A simple wired communication protocol — how ROS2 talks to Arduino over USB |
| Service (ROS2) | A request-response communication pattern (unlike topics which are broadcast) |
| SDF | Simulation Description Format — Gazebo's world file format |
| Topic (ROS2) | A named channel that nodes publish to or subscribe from |
| Trajectory | A sequence of joint positions over time — how smooth motion is commanded |
| URDF | Unified Robot Description Format — XML file that describes the robot's physical structure |
| Workspace | The folder containing all your ROS2 packages (src/, build/, install/) |
| Xacro | XML macro preprocessor for URDF — adds variables and templates to reduce repetition |
| YOLO | You Only Look Once — a family of real-time AI object detection models |
| YOLOv8 | The 8th generation YOLO model by Ultralytics — fast, accurate, easy to use in Python |

# SECTION 17 — Final Tips for Success

## 17.1  Development Order — Don't Skip Steps

Follow this exact order. Each step builds on the last:

- Step 1 — Set up Ubuntu 24.04 + ROS2 Jazzy + Gazebo Harmonic on your machine

- Step 2 — Get the base URDF (ferasboulala's 5-DOF arm) loading in RViz and Gazebo

- Step 3 — Add ros2_control tags and get the arm moving with joint_trajectory_controller

- Step 4 — Add a simulated camera to the URDF and get images publishing to a ROS2 topic

- Step 5 — Write the OpenCV colour detection node and test it against the simulated camera

- Step 6 — Write the autonomous sorting pipeline — detect object, compute coordinates, send MoveIt2 goal

- Step 7 — Write the local gesture control node using MediaPipe

- Step 8 — Write the Mode Manager that switches between autonomous and local gesture

- Step 9 — Build the phone-side MQTT HTML page and add remote gesture mode

- Step 10 — Add YOLOv8 as an upgrade to OpenCV colour detection

- Step 11 (Hardware) — Write the Arduino firmware and ros2_control hardware interface

## 17.2  Common Mistakes to Avoid

- Wrong ROS2 + Gazebo versions — Jazzy only works with Harmonic. Don't mix versions.

- Forgetting to source — Always run 'source install/setup.bash' after building, in every new terminal

- BGR vs RGB — OpenCV uses BGR (Blue-Green-Red). MediaPipe and many AI tools use RGB. Always convert with cv2.cvtColor(img, cv2.COLOR_BGR2RGB) before sending to AI models.

- Radians vs Degrees — ROS2 and URDF use radians. Python's math.pi = 3.14159 = 180°. Use math.radians(degrees) to convert.

- Joint names must match — The joint names in your URDF, your ros2_control YAML, and your controller must be exactly identical — one typo and the arm won't move.

- Thread safety — MQTT callbacks run in a separate thread from ROS2 callbacks. Use Python's threading.Lock() to protect shared data.

## 17.3  How to Debug

- ros2 topic list — Shows all active topics — use this to check if nodes are publishing

- ros2 topic echo /topic_name — Prints all messages on a topic in real time

- ros2 node list — Shows all running nodes

- ros2 run rqt_graph rqt_graph — Shows a visual graph of all nodes and their connections

- self.get_logger().info('msg') — The ROS2 way to print debug messages from inside a node

## 17.4  You've Got This!

This project is ambitious, but every concept in it is learnable by a motivated person. You now have a clear understanding of what every technology does and why it's there. The documentation gives you the code structure. This guide gives you the conceptual foundation.

Start with Week 1 — Python OOP. Build confidence one week at a time. When you get stuck, re-read the relevant section here, then search for the specific error message online. The ROS2 community is large and supportive.

Good luck building your robotic arm. 🦾

End of Beginner's Guide — Design & Development of a Dual-Mode Vision-Guided Robotic Arm